S-164 – Part 4

Chart Related Functions

Table of Contents

Pages

4.1	Mode and orientation	1

4.2	Scale bar	2

4.2.1	Display of scale bar	2

4.2.2	Display of scale bar	3

4.3	Feature information	3

4.4	Radar and Plotting Information	16

4.5	Accuracy	23

4.5.1	Distance and azimuth between geographical positions	23

4.5.2	Geographical position from a known position and distance/azimuth	24

4.5.3	Rhumb line distance and azimuth between geographical positions	25

4.5.4	Geodesics	26

4.5.5	Rhumb Lines	27

4.5.6	Positions for use in Accuracy Tests - Geodesics	29

4.5.7	Positions for use in Accuracy Tests – Rhumb Lines	35

4.6	Symbols	39

4.6.1	Symbol Size	39

4.6.2	Display of ECDIS chart 1 symbols of correct size	39

4.6.3	Size in pixels of the check symbol CHKSYM01	39

4.6.4	Display of text at the correct size	40

4.6.5	ECDIS Chart 1	40

4.6.6	Display redraw	40

4.7	Units and Legend	41

4.7.1	Legend – Multiple Product Specifications.	42

4.8	Other ECDIS and Chart Related Functionality	43

4.8.1	ECDIS Chart 1	43

4.8.2	Use of the Colour Differentiation Diagram	45

4.8.3	Data Quality Indicators and Portrayal	47

4.8.4	Miscellaneous encoding	47

Page intentionally left blank

## Mode and orientation

| Test Reference | ModeOrientation1 | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of the north arrow symbol. |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp |  |  |  |
| Action |  |  |  |
| Observe the display. If the EUT offers the capability to show other than north-up presentation; Change the presentation to non-north up and observe the display. |  |  |  |
| Results |  |  |  |
| Confirm that the north arrow symbol is always displayed at the top left corner of the chart area, not overlapping the scale or latitude bar when the scale bar extends to the full height of the display If the EUT supports changing to non-north up presentations confirm that the symbol realigns to north. |  |  |  |

| Test Reference | ModeOrientation2 | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| True motion operation. |  |  |  |
| Setup |  |  |  |
| As for ModeOrientation1 |  |  |  |
| Action |  |  |  |
| Ensure that true motion is provided. Reset the display and check that the generation of the neighbouring area takes place automatically at a distance selected by the Mariner. |  |  |  |
| Results |  |  |  |
| Confirm that true motion operation is provided and that the generation of the neighbouring area takes place automatically at a distance selected by the Mariner. |  |  |  |

| Test Reference | ModeOrientation3 | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Manual adjustment of chart display area and own ship position. |  |  |  |
| Setup |  |  |  |
| As for ModeOrientation |  |  |  |
| Action |  |  |  |
| Manually adjust the chart display area. Change the position of own ship relative to the edge of the display. |  |  |  |
| Results |  |  |  |
| Confirm that it is possible to change manually the chart area and the position of own ship relative to the edge of the display. |  |  |  |

| Test Reference | NoDataAvailable | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| No ENC data available. |  |  |  |
| Setup |  |  |  |
| As for ModeOrientation Ship position as follows: 32°27.88’S 061°20.66’E (an area with no ENC) |  |  |  |
| Action |  |  |  |
| Observe the display. |  |  |  |
| Results |  |  |  |
| Confirm that a “No ENC available” indication is provided. |  |  |  |

| Test Reference | NonNorthUp | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display in non ‘north-up’ orientation. |  |  |  |
| Setup |  |  |  |
| As for ModeOrientation |  |  |  |
| Action |  |  |  |
| For each bearing-stabilised orientation other than ‘north-up’ that may be provided, confirm by analytical evaluation that for turning rates between 0 deg/s and 20 deg/s the displayed chart symbols and text do not re-orient more often than 2 times per second and remain legible if they do not remain fixed. |  |  |  |
| Results |  |  |  |
| Confirm that the displayed symbols and text do not re-orient more often than 2 times per second and remain legible. The symbols and text may remaining fixed and in this case will not re-orientate. |  |  |  |

## Scale bar

### Display of scale bar

| Test Reference | ScaleBar | IHO Reference | S-98 12.10.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of scale bar at appropriate scales. |  |  |  |
| Setup |  |  |  |
| Load exchange set PowerUp Set Display Category Base Display. |  |  |  |
| Action |  |  |  |
| Zoom to a display scale greater than 1:80 000 (such as 1:25 000), observe the display. |  |  |  |
| Results |  |  |  |
| Confirm that a scale bar is displayed. Also confirm that the scale bar is displayed between 2mm and 4mm from the left side of the chart display area. |  |  |  |

### Display of scale bar

| Test Reference | LatitudeBar | IHO Reference | S-98 12.10.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of latitude bar at appropriate scales. |  |  |  |
| Setup |  |  |  |
| Load exchange set PowerUp Set Display Category Base Display. |  |  |  |
| Action |  |  |  |
| Zoom to a display scale less than 1:80 000 (such as 1:300 000), observe the display. |  |  |  |
| Results |  |  |  |
| Confirm that a latitude bar is displayed. Also confirm that the scale bar is displayed between 2mm and 4mm from the left side of the chart display area. |  |  |  |

## Feature information

| Test Reference | FeatureInformation1 | IHO Reference | S-98 15.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| General rules for cursor pick report |  |  |  |
| Setup |  |  |  |
| Load exchange set PowerUp Select Display Category Other. |  |  |  |
| Action |  |  |  |
| 1. Select several features of - depth area; - restricted area; - sea area; - depth contour; - ferry route; - recommended track; - buoy (for example buoy and light at 32°29.50’S - 061°00.46’E); - light; - wreck. 2. Observe feature information. 3. Remove feature information from display. |  |  |  |
| Results |  |  |  |

- The following rules shall be applied to the pick report: - Full feature and attribute names must be displayed. If a language pack is installed then the feature and attribute names must use the translated names contained in the language pack according to user preference – user language preferences are described in clause Erreur ! Source du renvoi introuvable. - Listed value names must be displayed instead of the numerical code. - There must not be any padding of attribute values, for example, a height of 10 metres must not be padded to 10.000000 metres as this could potentially confuse or mislead the Mariner. - Units of measure must be included after all attribute values when available. - Cursor pick report must extend to include both information associations and feature associations, including aggregations and compositions, which may link to information types or features that carry additional information such as a featureName. - Dates must be given in the form “Day Month Year” DD-MMM-YYYY. - Month abbreviations must be: JAN, FEB, MAR, APR, MAY, JUN, JUL, AUG, SEP, OCT, NOV, DEC. - The Pick Report must only return information about the visible features on the ECDIS display. If the viewing group is turned on all features, even “no symbol” features without visible presentation within that viewing group, must be provided in the cursor Pick Report, i.e. only active drawing instructions must be included. - Pick Report must include the information associated with the spatial object limited to the area of the cursor pick. For example, information related to accuracy. - Complex or Simple Attributes with private visibility must not be included. - Descriptions of attributes and their values, available in the feature catalogue, must be shown on demand. 2. Text associated with chart features must be removed from the display. Note: The text and background colour of pick report is specified by the OEM [This test needs to also include other data products (particularly S-124 where the area covered by NW is not portrayed]

| Test Reference | FeatureInformation2 | IHO Reference | S-98 15.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Pick report descriptions and sorting |  |  |  |
| Setup |  |  |  |
| As for test FeatureInformation1. |  |  |  |
| Action |  |  |  |
| Select several features as mentioned in 4.3 |  |  |  |
| Results |  |  |  |
| - A plain language explanation of each symbol shall be used as included in portrayal catalogue to provide quick and understandable information which is not always obvious from the feature class and attribute information. - Attribute values provided in addition to the above explanation shall be connected to their meaning, and the definitions shall also be available. - The feature information shall be sorted by the drawing priority of the feature as defined in the portrayal catalogue. When the drawing priority of features is equal, the geometric primitive shall be used to order the information (points followed by curves and finally surfaces). - Check that the content displayed in the pick report is configurable by the user. |  |  |  |

| Test Reference | FeatureInformation3 | IHO Reference | S-98 15.2.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| User defined cursor pick parameters, if available |  |  |  |
| Setup |  |  |  |
| As for test FeatureInformation1. |  |  |  |
| Action |  |  |  |
| 1. Configure the cursor pick parameter as available. 2. Select several features as mentioned in 4.3 |  |  |  |
| Results |  |  |  |
| 1. The cursor pick parameters may be configurable by the user and available for presentation. 2. The content of the pick report shall be presented as configured. |  |  |  |

| Test Reference |  | FeatureInformation4 |  | IHO Reference |  | S-98 15.1 |
| --- | --- | --- | --- | --- | --- | --- |
| Test description |  |  |  |  |  |  |
| Hover-over function for feature information (optional) Test shall only be performed if a hover-over function for feature information is provided. |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |
| As for test FeatureInformation1. |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |
| 1. Configure the hover-over function OFF. 2. Move cursor to one of the features in the table below and to features where additional information is available or date dependent features: 3. Configure the hover-over function ON. 4. Move cursor to one of the features mentioned in 2. 5. Move cursor to any other features. |  |  |  |  |  |  |
|  | Features |  | S-101 Acronym |  |  |  |
|  | Lights |  | AllRoundLight |  |  |  |
|  | Beacon, cardinal |  | BuoyCardinal |  |  |  |
|  | Beacon, isolated danger |  | BuoyIsolatedDAnger |  |  |  |
|  | Beacon, lateral |  | BeaconLateral |  |  |  |
|  | Beacon, safe water |  | BeaconSafeWater |  |  |  |
|  | Beacon, special purpose/general |  | BeaconSpecialPurpose |  |  |  |
|  | Buoy, cardinal |  | BuoyCardinal |  |  |  |
|  | Buoy, installation |  | BuoyInstallation |  |  |  |
|  | Buoy, isolated danger |  | BuoyIsolatedDanger |  |  |  |
|  | Buoy, lateral |  | BuoyLateral |  |  |  |
|  | Buoy, safe water |  | BuoySafeWater |  |  |  |
|  | Buoy, special purpose/general |  | BuoySpecialPurpose |  |  |  |
|  | Landmarks |  | Landmark |  |  |  |
| Results |  |  |  |  |  |  |
| 1. It shall be possible to switch OFF the hover-over function. 2. There shall be no information of chart features displayed when hovering over it. 3. It shall be possible to switch ON the hover-over function. 4. Important information of chart features shall be displayed when hovering over it. 5. When hovering over other chart features no information shall be displayed. |  |  |  |  |  |  |

| Test Reference | FeatureInformation5 | IHO Reference | S-98 15.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Presentation of unknown attributes There is no generic special presentation for unknown attributes. Some presentations may indicate question mark, but that is because something mandatory is missing for the feature. The main purpose of this test is to check that ECDIS is able to accept ENC datasets which contain unknown attributes. The real use case is when ECDIS is not upgraded for latest IHO standard and therefore the ECDIS does not understand all attributes. |  |  |  |
| Setup |  |  |  |
| Load the exchange set InvalidFeatures dataset 101AA00INVOB.000 : - Select Display Category Other - Set the Safety contour value to 0 m - Select Symbolized Boundaries - Set symbolized symbols to Off |  |  |  |

Action

Select chart features with unknown attribute for cursor pick report.

Results

Check ENC symbols shown in the ECDIS against the corresponding graphical plot. Select one by one each of 6 features for cursor pick report. The result of cursor pick shall be: a) Wreck with attribute Water level effect (covers and uncovers); b) Obstruction with attribute Value of Sounding (no value) and Water Level Effect (no value); c) Restricted area without any attribute; d) Buoy, cardinal with attributes Buoy shape (spar (spindle)), Category of cardinal mark (north cardinal mark), Colour, Colour pattern (horizontal stripes), Topmark and UnknownAtt2; e) Cable, submarine with attribute UnknownAtt1; f) Silo/Tank with attribute UnknownAtt2.

TBD

| Test Reference | InteroperabilityIdentifer | IHO Reference | 15.1 (10) |
| --- | --- | --- | --- |
| Test description |  |  |  |
| TODO: - Test pick report function where the feature picked has an interoperabilityIdentifier which is referred to by an S-124 featureReference.interoperabilityIdentifier. This tests that NW can link to ENC features. - This is accomplished by setting interoperabilityIdentifier in S-101 features then setting featureReference.interoperabilityIdentifier in S-124 to the same value and executing a pick report on the S-101. |  |  |  |

| Test Reference | TidalStreamPanelData | IHO Reference | S-98 15.4 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of tidal stream panel Data |  |  |  |
| Setup |  |  |  |
| Load exchange set PowerUp |  |  |  |
| Action |  |  |  |
| 1. Select an example of TidalStreamPanelData (tidal stream panel information) 1a. select the complex attribute tidal stream panel values at 32°31.45’S 60°56.35’E for display; 2. Select an example of TidalAStreamPanelData (tidal stream prediction by harmonic methods) 2a. select tidal stream prediction by harmonic methods feature at 32°32.57’S 60°57.69’E for display; 3. Repeat step 1 and 2 for different light conditions (DAY, DUSK, NIGHT). |  |  |  |

Results

1a. The data must be displayed in a way that it can be easily read and is logically presented, in a format as follows: TODO: replace with S-98 Updated Image.

3. The data must be displayed as appropriate for the selected light condition (DAY, DUSK, NIGHT).

| Test Reference | Tidal panels (2) | IHO Reference | S-98 15.4 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Show multiple tidal panels when more than one is included in the feature. |  |  |  |

| Test Reference | SupplementaryFile2 | IHO Reference | S-98 15.2.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of supplementary text file |  |  |  |
| Setup |  |  |  |
| As for test FeatureInformation1. |  |  |  |
| Action |  |  |  |
| 1. Select an example of a note encoded using information attributes (for example Caution Area at approximately 32°34.74’S 061°08.92’E); 2. Repeat step 1 for different light conditions (DAY, DUSK, NIGHT). |  |  |  |
| Results |  |  |  |
| 1. The note must be displayed within the light level of the current display and in a way that it can be easily read, for example by displaying the note as it might appear on a paper chart (for example content of 10100AAIECTMP.TXT file as contained in the directory of loaded ENCs). 2. The note must be displayed as appropriate for the selected light condition (DAY, DUSK, NIGHT). 3. The content of the note must commence at the location specified by the fileLocator reference, as shown in the image |  |  |  |

Example of Text 101AA00IECTMP.TXT over cell 10100AA_X0000.000, Day palette tbd

Example of Text 101AA00IECTMP.TXT over cell 10100AA_X0000.000, Dusk palette TBD

Example of Text 101AA00IECTMP.TXT over cell 10100AA_X0000.000, Night palette tbd

| Test Reference | PictorialRepresentation | IHO Reference | S-98 15.2.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of picture representation |  |  |  |
| Setup |  |  |  |
| As for test FeatureInformation |  |  |  |
| Action |  |  |  |
| 1. Select an example of the attribute pictorialRepresentation 1a. select landmark feature at 32°31.95’S 60°54.34’E and select picture representation for display; 1b. select area feature of 32°30.25’S 60°54.64’E with NauticalInformation and select picture representation for display; 2. Repeat step 1a and b for different light conditions (DAY, DUSK, NIGHT). |  |  |  |
| Results |  |  |  |
| 1a. The picture 101AA00TESTPC.TIF must be displayed; 1b. The picture 101AA00X4000T.TIF must be displayed; 2. The pictures must be displayed as appropriate for the selected light condition (DAY, DUSK, NIGHT). It shall not affect the user’s night vision. |  |  |  |
| Example of Picture 101AA00TESTPC.TIF over cell 10100AA_X0000.000, Day palette TBD |  |  |  |

Example of Picture 101AA00TESTPC.TIF over cell 10100AA_X0000.000, Dusk palette TBD

Example of Picture 101AA00TESTPC.TIF over cell 10100AA_X0000.000, Night palette TBD

Example of Picture 101AA00X4000T.TIF over cell 10100AA_X0000.000, Day palette TBD

## Radar and Plotting Information

Where the capability for displaying radar or radar tracks is provided, in addition to the requirements of IEC 62288 for radar displays and presentation of target information, perform the following:

| Test Reference |  | RadarOverlay |  |  |  | IHO Reference |  |  | S-98 9.2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| Display of Radar overlays with System Database information |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| Load exchange set PowerUp Display cell 10100AA_X01NE at 3 NM range scale - Select Safety contour value to 8 m - Select Safety depth value to 8 m - Select Plain boundaries Set symbolized symbols to Off.. |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Switch on the following (where available): - Radar image overlay - Radar tracked target information AIS information |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| Confirm by observation that same System Database features are under or over radar echoes as in the example pictures. Note that some examples contain intentionally a lot of radar echo noise in order to give many examples of the System Database features which shall be over or under radar echoes Day with radar tracked targets. Display Category Display Base + Lights TBD |  |  |  |  |  |  |  |  |  |  |

Dusk with radar tracked targets. Display Category Display Base + Lights TBD

Day with radar echoes and tracked targets. Display Category Display Base + Lights TBD

Dusk with radar echoes and tracked targets. Display Category Display Base + Lights TBD

Day with very noisy radar echoes and tracked targets. Display Category Other, Select Highlight info, Select Shallow water dangers. Note: This example clearly shows which SYSTEM DATABASE features are above radar echoes TBD

Dusk with very noisy radar echoes and tracked targets. Display Category Other, Select Highlight info, Select Shallow water dangers. Note: This example clearly shows which SYSTEM DATABASE features are above radar echoes TBD

## Accuracy

Note:

In this section calculations are based on the WGS-84 spheroid:

Semi-major axis       6378137.0000m

Semi-minor axis       6356752.3142m

Eccentricity squared 0.00669437999013

Flattening                 298.257223563

The WGS-84 spheroid is defined by its semi-major axis and flattening 1/f = 1/298.257223563.

The other parameters are derived from a and f.

Conversion of metres (m) to nautical miles (NM) uses

1 NM = 1852 m.

The tests contained within this section shall be executed using the Electronic Bearing Line (EBL) and Variable Range Marker (VRM) tools provided by the ECDIS system.

The tolerance for distances is 1% or 30m whichever is greater. The tolerance for bearings is 1º.

The positions used in this section are also included in the files “4.6 Accuracy-Geodesic.doc” and “4.6 Accuracy-Rhumb Lines.doc” in the “4.6 Accuracy” folder within the TDS.

### Distance and azimuth between geographical positions

| Test Reference | Accuracy1 | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| True distance and azimuth between two geographical positions a). |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Viking 49/27-B 32º35.224’S 061º17.710’E Corund Cape Light 32º27.436’S 060º58.609’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 33193.554 m / 17.9231 NM Bearing from Viking 49/27-B to Corund Cape Light is 295.614 degrees Bearing from Corund Cape Light to Viking 49/27-B is 115.785 degrees |  |  |  |

| Test Reference | Accuracy2 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| True distance and azimuth between two geographical positions b). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Viking 49/27-B 32º35.224’S 061º17.710’E Castlerigg Light 32º23.280’S 060º58.496’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 37326.351 m / 20.1546 NM Bearing from Viking 49/27-B to Castlerigg Light is 306.172 degrees Bearing from Castlerigg Light to Viking 49/27-B is 126.344 degrees |  |  |  |

| Test Reference | Accuracy3 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| True distance and azimuth between two geographical positions c). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Corund Cape Light 32º27.447’S 060º58.599’E Worm Head Light 32º31.958’S 060º54.337’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 10680.859 m / 5.7672 NM Bearing from Corund Cape Light to Worm Head Light is 218.665 degrees Bearing from Worm Head Light to Corund Cape Light is 38.703 degrees |  |  |  |

### Geographical position from a known position and distance/azimuth

| Test Reference | Accuracy4 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geographical position from known position and distance/azimuth a). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1. |  |  |  |
| Action |  |  |  |
| From the following position: Viking 49/27-B 32º35.224’S 061º17.710’E Enter a distance and bearing of: True Distance 33193.554 m / 17.9231 NM Bearing 295.614 degrees |  |  |  |
| Results |  |  |  |
| Confirm that the end geographical position is: Corund Cape Light 32º27.436’S 060º58.609’E |  |  |  |

| Test Reference | Accuracy5 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geographical position from known position and distance/azimuth b). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| From the following position: Viking 49/27-B 32º35.224’S 061º17.710’E Enter a distance and bearing of: True Distance 37326.351 m / 20.1546 NM Bearing 306.172 degrees |  |  |  |
| Results |  |  |  |
| Confirm that the end geographical position is: Castlerigg Light 32º23.280’S 060º58.496’E |  |  |  |

| Test Reference | Accuracy6 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geographical position from known position and distance/azimuth c). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| From the following position: Corund Cape Light 32º27.447’S 060º58.599’E Enter a distance and bearing of: True Distance 10680.859 m / 5.7672 NM Bearing 218.665 degrees |  |  |  |
| Results |  |  |  |
| Confirm that the end geographical position is: Worm Head Light 32º 31.958’S 60º 54.337’E |  |  |  |

### Rhumb line distance and azimuth between geographical positions

| Test Reference | Accuracy7 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb line distance and azimuth between two geographical positions a). |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Viking 49/27-B 32º35.224’S 061º17.710’E Corund Cape Light 32º27.436’S 060º58.609’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 33193.567 m / 17.9231 NM Bearing from Viking 49/27-B to Corund Cape Light is 295.699 degrees Bearing from Corund Cape Light to Viking 49/27-B is 115.699 degrees |  |  |  |

| Test Reference | Accuracy8 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb line distance and azimuth between two geographical positions b). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Viking 49/27-B 32º35.224’S 061º17.710’E Castlerigg Light 32º23.280’S 060º58.496’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 37326.365 m / 20.1546 NM Bearing from Viking 49/27-B to Castlerigg Light is 306.258 degrees Bearing from Castlerigg Light to Viking 49/27-B is 126.258 degrees |  |  |  |

| Test Reference | Accuracy9 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb line distance and azimuth between two geographical positions c). |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Measure the distance and azimuth between the following two features: Corund Cape Light 32º27.447’S 060º58.599’E Worm Head Light 32º31.958’S 060º54.337’E |  |  |  |
| Results |  |  |  |
| Confirm that the results are as follows: True Distance 10680.859 m / 5.7672 NM Bearing from Corund Cape Light to Worm Head Light is 218.684 degrees Bearing from Worm Head Light to Corund Cape Light is 38.684 degrees |  |  |  |

### Geodesics

| Test Reference | Accuracy10 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geodesic lines and circle, northern quadrant. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 2-6 of the positions listed in section “Positions for use in Accuracy Tests – Geodetics”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions and that the Geodesic circle corresponds to range rings at 2 000 000 m intervals. |  |  |  |

| Test Reference | Accuracy11 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geodesic lines and circle, crossing the equator. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 7-11 of the positions listed in section “Positions for use in Accuracy Tests – Geodetics”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions and that the Geodesic circle corresponds to range rings at 2 000 000 m intervals. |  |  |  |

| Test Reference | Accuracy12 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Geodesic lines southern quadrant. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 12-16 of the positions listed in section “Positions for use in Accuracy Tests – Geodetics”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions and that the Geodesic circle corresponds to range rings at 2 000 000 m intervals. |  |  |  |

### Rhumb Lines

| Test Reference | Accuracy13 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb lines, northern quadrant. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1 |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 2-5 of the positions listed in section “Positions for use in Accuracy Tests – Rhumb Lines”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions. |  |  |  |

| Test Reference | Accuracy14 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb lines, crossing the equator. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1. |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 6-9 of the positions listed in section “Positions for use in Accuracy Tests – Rhumb Lines”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions. |  |  |  |

| Test Reference | Accuracy15 | IHO Reference | - |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Rhumb lines, southern quadrant. |  |  |  |
| Setup |  |  |  |
| As for test Accuracy1. |  |  |  |
| Action |  |  |  |
| Plot positions listed in sets 10-13 of the positions listed in section “Positions for use in Accuracy Tests – Rhumb Lines”. |  |  |  |
| Results |  |  |  |
| Confirm that the lines drawn pass through or sufficiently close to the listed positions. |  |  |  |

### Positions for use in Accuracy Tests - Geodesics

The following sections contain a series of latitudes and longitudes which define a number of geodesics.

These points are intended to allow type approval authorities to test the ability of ECDIS to calculate geodesics correctly.

Conversion of metres (m) to nautical miles (NM) uses

1 NM = 1852 m.

Set 1 Micklefirth

Usage Band 4

Viking 49/27-B     32º35.224S  061º17.710E

Corund Cape Light  32º27.436S  060º58.609E

True Distance      33193.554 m / 17.9231 NM

Forward Bearing    295.614 degrees

Reverse Bearing    115.785 degrees

Viking 49/27-B     32º35.224S  061º17.710E

Castlerigg Light   32º23.280S  060º58.496E

True Distance      37326.351 m / 20.1546 NM

Forward Bearing    306.172 degrees

Reverse Bearing    126.344 degrees

Usage Band 5

Corund Cape Light  32º27.447S  060º58.599E

Worm Head Light    32º31.958S  060º54.337E

True Distance      10680.859 m / 5.7672 NM

Forward Bearing    218.665 degrees

Reverse Bearing    38.703 degrees

Long Geodesics - North West Quadrant.

Set 2 Long Diagonal (30ºN, 60ºW to 60ºN, 30ºW)

Point1        30º00.0000N     060º00.0000W

Point2        31º38.1452N     059º05.9571W

Point3        33º15.8706N     058º09.9924W

Point4        34º53.1348N     057º11.9156W

Point5        36º29.8923N     056º11.5178W

Point6        38º06.0926N     055º08.5692W

Point7        39º41.6796N     054º02.8166W

Point8        41º16.5909N     052º53.9805W

Point9        42º50.7564N     051º41.7515W

Point10      44º24.0976N     050º25.7868W

Point11      45º56.5257N     049º05.7067W

Point12      47º27.9409N     047º41.0895W

Point13      48º58.2294N     046º11.4681W

Point14      50º27.2626N     044º36.3244W

Point15      51º54.8937N     042º55.0855W

Point16      53º20.9554N     041º07.1195W

Point17      54º45.2565N     039º11.7330W

Point18      56º07.5789N     037º08.1699W

Point19      57º27.6730N     034º55.6135W

Point20      58º45.2547N     032º33.1935W

Point21      60º00.0000N     030º00.0000W

Set 3 Long Diagonal (30ºN, 30ºW to 60ºN, 60ºW)

Point1        30º00.0000N     030º00.0000W

Point2        31º38.1452N     030º54.0429W

Point3        33º15.8706N     031º50.0076W

Point4        34º53.1348N     032º48.0844W

Point5        36º29.8923N     033º48.4822W

Point6        38º06.0926N     034º51.4308W

Point7        39º41.6796N     035º57.1833W

Point8        41º16.5909N     037º06.0195W

Point9        42º50.7564N     038º18.2485W

Point10      44º24.0976N     039º34.2132W

Point11      45º56.5257N     040º54.2933W

Point12      47º27.9409N     042º18.9105W

Point13      48º58.2294N     043º48.5319W

Point14      50º27.2626N     045º23.6756W

Point15      51º54.8937N     047º04.9145W

Point16      53º20.9554N     048º52.8805W

Point17      54º45.2565N     050º48.2670W

Point18      56º07.5789N     052º51.8301W

Point19      57º27.6730N     055º04.3865W

Point20      58º45.2547N     057º26.8065W

Point21      60º00.0000N     060º00.0000W

Set 4 Long Horizontal (45ºN, 60ºW to 45ºN, 30ºW)

Point1        45º00.0000N     060º00.0000W

Point2        45º11.2519N     058º31.7916W

Point3        45º21.3608N     057º03.0317W

Point4        45º30.3133N     055º33.7738W

Point5        45º38.0973N     054º04.0740W

Point6        45º44.7022N     052º33.9908W

Point7        45º50.1188N     051º03.5849W

Point8        45º54.3397N     049º32.9185W

Point9        45º57.3588N     048º02.0555W

Point10      45º59.1720N     046º31.0608W

Point11      45º59.7767N     045º00.0000W

Point12      45º59.1720N     043º28.9392W

Point13      45º57.3588N     041º57.9446W

Point14      45º54.3397N     040º27.0815W

Point15      45º50.1188N     038º56.4152W

Point16      45º44.7022N     037º26.0092W

Point17      45º38.0973N     035º55.9260W

Point18      45º30.3133N     034º26.2263W

Point19      45º21.3608N     032º56.9684W

Point20      45º11.2519N     031º28.2085W

Point21      45º00.0000N     030º00.0000W

Set 5 Long Vertical (30ºN, 45ºW to 60ºN, 45ºW)

The geodesic runs along the 45ºW meridian.

Set 6 Circle (Centre 45ºN, 45ºW Radius 2 000 000 m Points every 15 degrees)

Point1        62º58.1482N      045º00.0000W

Point2        62º02.9175N      035º13.1324W

Point3        59º29.7703N      027º21.3716W

Point4        55º47.3417N      022º13.6842W

Point5        51º25.6105N      019º41.1668W

Point6        46º49.0062N      019º14.2861W

Point7        42º16.1548N      020º24.1958W

Point8        38º1.4970N        022º48.2871W

Point9        34º16.6609N      026º09.5368W

Point10      31º11.2085N      030º14.5458W

Point11      28º52.8672N      034º51.8044W

Point12      27º27.4359N      039º50.5197W

Point13      26º58.5455N      045º00.0000W

Point14      27º27.4359N      050º09.4803W

Point15      28º52.8672N      055º08.1956W

Point16      31º11.2085N      059º45.4542W

Point17      34º16.6609N      063º50.4632W

Point18      38º01.4970N      067º11.7129W

Point19      42º16.1548N      069º35.8042W

Point20      46º49.0062N      070º45.7139W

Point21      51º25.6105N      070º18.8332W

Point22      55º47.3417N      067º46.3158W

Point23      59º29.7703N      062º38.6284W

Point24      62º02.9175N      054º46.8676W

Point25      62º58.1482N      045º00.0000W

Long Geodesics (Crossing Equator).

Set 7 Long Diagonal (15ºN, 60ºW to 15ºS, 30ºW)

Point1        15º00.0000N     060º00.0000W

Point2        13º31.8194N     058º26.4185W

Point3        12º03.0524N     056º53.9818W

Point4        10º33.7708N     055º22.5552W

Point5        09º04.0440N     053º52.0065W

Point6        07º33.9393N     052º22.2057W

Point7        06º03.5224N     050º53.0251W

Point8        04º32.8574N     049º24.3384W

Point9        03º02.0073N     047º56.0210W

Point10      01º31.0343N     046º27.9492W

Point11      00º00.0000N     045º00.0000W

Point12      01º31.0343S     043º32.0508W

Point13      03º02.0073S     042º03.9789W

Point14      04º32.8574S     040º35.6615W

Point15      06º03.5224S     039º06.9749W

Point16      07º33.9393S     037º37.7942W

Point17      09º04.0440S     036º07.9935W

Point18      10º33.7708S     034º37.4447W

Point19      12º03.0524S     033º06.0182W

Point20      13º31.8194S     031º33.5815W

Point21      15º00.0000S     030º00.0000W

Set 8 Long Diagonal (15ºN, 30ºW to 15ºS, 60ºW)

Point1        15º00.0000N    030º00.0000W

Point2        13º31.8194N    031º33.5815W

Point3        12º03.0524N    033º06.0182W

Point4        10º33.7708N    034º37.4448W

Point5        09º04.0440N    036º07.9935W

Point6        07º33.9393N    037º37.7943W

Point7        06º03.5224N    039º06.9749W

Point8        04º32.8574N    040º35.6616W

Point9        03º02.0073N    042º03.9790W

Point10      01º31.0343N    043º32.0508W

Point11      00º00.0000N    045º00.0000W

Point12      01º31.0343S    046º27.9492W

Point13      03º02.0073S    047º56.0211W

Point14      04º32.8574S    049º24.3385W

Point15      06º03.5224S    050º53.0251W

Point16      07º33.9393S    052º22.2058W

Point17      09º04.0440S    053º52.0065W

Point18      10º33.7708S    055º22.5553W

Point19      12º03.0524S    056º53.9819W

Point20      13º31.8194S    058º26.4185W

Point21      15º00.0000S    060º00.0000W

Set 9 Long Horizontal (0ºN, 60ºW to 0ºN, 30ºW)

The geodesic runs along the Equator.

Set 10 Long Vertical (15ºS, 45ºW to 15ºN, 45ºW)

The geodesic runs along the 45ºW meridian.

Set 11 Circle (Centre 0ºN, 45ºW Radius 2 000 000 m Points every 15 degrees)

Point1         18º04.8887N     045º00.0000W

Point2         17º26.7433N     040º12.0936W

Point3         15º35.6306N     035º47.3375W

Point4         12º40.8191N     032º05.0570W

Point5         08º55.8234N     029º18.7826W

Point6         04º36.5608N     027º36.4877W

Point7         00º00.0000N     027º02.0217W

Point8         04º36.5608S     027º36.4877W

Point9         08º55.8234S     029º18.7826W

Point10       12º40.8191S     032º05.0570W

Point11       15º35.6306S     035º47.3375W

Point12       17º26.7433S     040º12.0936W

Point13       18º04.8887S     045º00.0000W

Point14       17º26.7433S     049º47.9064W

Point15       15º35.6306S     054º12.6625W

Point16       12º40.8191S     057º54.9430W

Point17       08º55.8234S     060º41.2174W

Point18       04º36.5608S     062º23.5123W

Point19       00º00.0000N     062º57.9783W

Point20       04º36.5608N     062º23.5123W

Point21       08º55.8234N     060º41.2174W

Point22       12º40.8191N     057º54.9430W

Point23       15º35.6306N     054º12.6625W

Point24       17º26.7433N     049º47.9064W

Point25       18º04.8887N     045º00.0000W

Long Geodesics - South West Quadrant.

Set 12 Long Diagonal (30ºS, 60ºW to 60ºS, 30ºW)

Point1        30º00.0000S      060º00.0000W

Point2        31º38.1452S      059º05.9571W

Point3        33º15.8706S      058º09.9924W

Point4        34º53.1348S      057º11.9156W

Point5        36º29.8923S      056º11.5178W

Point6        38º06.0926S      055º08.5692W

Point7        39º41.6796S      054º02.8166W

Point8        41º16.5909S      052º53.9805W

Point9        42º50.7564S      051º41.7515W

Point10      44º24.0976S      050º25.7868W

Point11      45º56.5257S      049º05.7067W

Point12      47º27.9409S      047º41.0895W

Point13      48º58.2294S      046º11.4681W

Point14      50º27.2626S      044º36.3244W

Point15      51º54.8937S      042º55.0855W

Point16      53º20.9554S      041º07.1195W

Point17      54º45.2565S      039º11.7330W

Point18      56º07.5789S      037º08.1699W

Point19      57º27.6730S      034º55.6135W

Point20      58º45.2547S      032º33.1935W

Point21      60º00.0000S      030º00.0000W

Set 13 Long Diagonal (30ºS, 30ºW to 60ºS, 60ºW)

Point1        30º00.0000S     030º00.0000W

Point2        31º38.1452S     030º54.0429W

Point3        33º15.8706S     031º50.0076W

Point4        34º53.1348S     032º48.0844W

Point5        36º29.8923S     033º48.4822W

Point6        38º06.0926S     034º51.4308W

Point7        39º41.6796S     035º57.1833W

Point8        41º16.5909S     037º06.0195W

Point9        42º50.7564S     038º18.2485W

Point10      44º24.0976S     039º34.2132W

Point11      45º56.5257S     040º54.2933W

Point12      47º27.9409S     042º18.9105W

Point13      48º58.2294S     043º48.5319W

Point14      50º27.2626S     045º23.6756W

Point15      51º54.8937S     047º04.9145W

Point16      53º20.9554S     048º52.8805W

Point17      54º45.2565S     050º48.2670W

Point18      56º7.5789S       052º51.8301W

Point19      57º27.6730S     055º04.3865W

Point20      58º45.2547S     057º26.8065W

Point21      60º00.0000S     060º00.0000W

Set 14 Long Horizontal (45ºS, 60ºW to 45ºS, 30ºW)

Point1        45º00.0000S     060º00.0000W

Point2        45º11.2519S     058º31.7916W

Point3        45º21.3608S     057º03.0317W

Point4        45º30.3133S     055º33.7738W

Point5        45º38.0973S     054º04.0740W

Point6        45º44.7022S     052º33.9908W

Point7        45º50.1188S     051º03.5849W

Point8        45º54.3397S     049º32.9185W

Point9        45º57.3588S     048º02.0555W

Point10      45º59.1720S     046º31.0608W

Point11      45º59.7767S     045º00.0000W

Point12      45º59.1720S     043º28.9392W

Point13      45º57.3588S     041º57.9446W

Point14      45º54.3397S     040º27.0815W

Point15      45º50.1188S     038º56.4152W

Point16      45º44.7022S     037º26.0092W

Point17      45º38.0973S     035º55.9260W

Point18      45º30.3133S     034º26.2263W

Point19      45º21.3608S     032º56.9684W

Point20      45º11.2519S     031º28.2085W

Point21      45º00.0000S     030º00.0000W

Set 15 Long Vertical (30ºS, 45ºW to 60ºS, 45ºW)

The geodesic runs along the 45ºW meridian.

Set 16 Circle (Centre 45ºS, 45ºW Radius 2 000 000 m Points every 15 degrees)

Point1        62º58.1482S      045º00.0000W

Point2        62º02.9175S      035º13.1324W

Point3        59º29.7703S      027º21.3716W

Point4        55º47.3417S      022º13.6842W

Point5        51º25.6105S      019º41.1668W

Point6        46º49.0062S      019º14.2861W

Point7        42º16.1548S      020º24.1958W

Point8        38º01.4970S      022º48.2871W

Point9        34º16.6609S      026º09.5368W

Point10      31º11.2085S      030º14.5458W

Point11      28º52.8672S      034º51.8044W

Point12      27º27.4359S      039º50.5197W

Point13      26º58.5455S      045º00.0000W

Point14      27º27.4359S      050º09.4803W

Point15      28º52.8672S      055º08.1956W

Point16      31º11.2085S      059º45.4542W

Point17      34º16.6609S      063º50.4632W

Point18      38º01.4970S      067º11.7129W

Point19      42º16.1548S      069º35.8042W

Point20      46º49.0062S      070º45.7139W

Point21      51º25.6105S      070º18.8332W

Point22      55º47.3417S      067º46.3158W

Point23      59º29.7703S      062º38.6284W

Point24      62º02.9175S      054º46.8676W

Point25      62º58.1482S      045º00.0000W

### Positions for use in Accuracy Tests – Rhumb Lines

The following sections contain a series of latitudes and longitudes which define a number of rhumb lines. These points are intended to allow type approval authorities to test the ability of ECDIS to calculate rhumb lines correctly.

All calculations are based on the WGS-84 spheroid:

Semi-major axis		6378137.0000m

Semi-minor axis 	6356752.3142m

Eccentricity squared	0.0066943800

Flattening		298.25722356

Conversion of metres (m) to nautical miles (NM) uses

1 NM = 1852 m.

Set 1 – not applicable

Long Rhumb Lines - North West Quadrant.

Set 2 Long Diagonal (30ºN, 30ºW to 60ºN, 60ºW)

Point1		30º00.0000N	030º00.0000W

Point2		31º30.2165N	031º11.4806W

Point3		33º00.4119N	032º24.1146W

Point4		34º30.5854N	033º37.9913W

Point5		36º00.7368N	034º53.2065W

Point6		37º30.8656N	036º09.8628W

Point7		39º00.9713N	037º28.0713W

Point8		40º31.0539N	038º47.9519W

Point9		42º01.1129N	040º09.6347W

Point10	43º31.1484N	041º33.2615W

Point11	45º01.1601N	042º58.9871W

Point12	46º31.1481N	044º26.9812W

Point13	48º01.1124N	045º57.4306W

Point14	49º31.0531N	047º30.5417W

Point15	51º00.9704N	049º06.5435W

Point16	52º30.8645N	050º45.6910W

Point17	54º00.7358N	052º28.2698W

Point18	55º30.5845N	054º14.6010W

Point19	57º00.4111N	056º05.0479W

Point20	58º30.2161N	058º00.0234W

Point21	60º00.0000N	060º00.0000W

Set 3 Long Diagonal (60ºN, 30ºW to 30ºN, 60ºW)

Point1		60º00.0000N	030º00.0000W

Point2		58º30.2161N	031º59.9767W

Point3		57º00.4111N	033º54.9521W

Point4		55º30.5845N	035º45.3990W

Point5		54º00.7358N	037º31.7302W

Point6		52º30.8645N	039º14.3090W

Point7		51º00.9704N	040º53.4565W

Point8		49º31.0531N	042º29.4583W

Point9		48º01.1124N	044º02.5694W

Point10	46º31.1481N	045º33.0188W

Point11	45º01.1601N	047º01.0129W

Point12	43º31.1484N	048º26.7385W

Point13	42º01.1129N	049º50.3653W

Point14	40º31.0539N	051º12.0481W

Point15	39º00.9713N	052º31.9287W

Point16	37º30.8656N	053º50.1372W

Point17	36º00.7368N	055º06.7935W

Point18	34º30.5854N	056º22.0087W

Point19	33º00.4119N	057º35.8854W

Point20	31º30.2165N	058º48.5194W

Point21	30º00.0000N	060º00.0000W

Set 4 Long Horizontal (45ºN, 60ºW to 45ºN, 30ºW)

The rhumb line runs along the 45ºN parallel.

Set 5 Long Vertical (30ºN, 45ºW to 60ºN, 45ºW)

The rhumb line runs along the 45ºW meridian.

Long Rhumb Lines (Crossing Equator).

Set 6 Long Diagonal (15ºN, 60ºW to 15ºS, 30ºW)

Point1		15º00.0000N	060º00.0000W

Point2		13º30.0344N	058º28.2185W

Point3		12º00.0581N	056º57.0084W

Point4		10º30.0722N	055º26.3012W

Point5		 09º00.0778N	053º56.0303W

Point6		 07º30.0761N	052º26.1306W

Point7		 06º00.0683N	050º56.5384W

Point8		 04º30.0555N	049º27.1908W

Point9		 03º00.0391N	047º58.0260W

Point10	 01º30.0202N	046º28.9826W

Point11	 00º00.0000N	045º00.0000W

Point12	 01º30.0202S	043º31.0173W

Point13	 03º00.0391S	042º01.9740W

Point14	 04º30.0555S	040º32.8092W

Point15	 06º00.0683S	039º03.4616W

Point16	 07º30.0761S	037º33.8694W

Point17	 09º00.0778S	036º03.9697W

Point18	10º30.0722S	034º33.6988W

Point19	12º00.0581S	033º02.9916W

Point20	13º30.0344S	031º31.7815W

Point21	15º00.0000S	030º00.0000W

Set 7 Long Diagonal (15ºN, 30ºW to 15ºS, 60ºW)

Point1		15º00.0000N	030º00.0000W

Point2		13º30.0344N	031º31.7815W

Point3		12º00.0581N	033º02.9916W

Point4		10º30.0722N	034º33.6988W

Point5		 09º00.0778N	036º03.9697W

Point6		 07º30.0761N	037º33.8694W

Point7		 06º00.0683N	039º03.4616W

Point8		 04º30.0555N	040º32.8092W

Point9		 03º00.0391N	042º01.9740W

Point10	 01º30.0202N	043º31.0174W

Point11	 00º00.0000N	045º00.0000W

Point12	 01º30.0202S	046º28.9827W

Point13	 03º00.0391S	047º58.0260W

Point14	 04º30.0555S	049º27.1908W

Point15	 06º00.0683S	050º56.5384W

Point16	 07º30.0761S	052º26.1306W

Point17	 09º00.0778S	053º56.0303W

Point18	10º30.0722S	055º26.3012W

Point19	12º00.0581S	056º57.0084W

Point20	13º30.0344S	058º28.2185W

Point21	15º00.0000S	060º00.0000W

Set 8 Long Horizontal (0ºN, 60ºW to 0ºN, 30ºW)

The rhumb line runs along the Equator.

Set 9 Long Vertical (15ºS, 45ºW to 15ºN, 45ºW)

The rhumb line runs along the 45ºW meridian.

Long Rhumb Lines - South West Quadrant.

Set 10 Long Diagonal (30ºS, 30ºW to 60ºS, 60ºW)

Point1		30º00.0000S	030º00.0000W

Point2		31º30.2165S	031º11.4806W

Point3		33º00.4119S	032º24.1146W

Point4		34º30.5854S	033º37.9913W

Point5		36º00.7368S	034º53.2065W

Point6		37º30.8656S	036º09.8628W

Point7		39º00.9713S	037º28.0713W

Point8		40º31.0539S	038º47.9519W

Point9		42º01.1129S	040º09.6347W

Point10	43º31.1484S	041º33.2615W

Point11	45º01.1601S	042º58.9871W

Point12	46º31.1481S	044º26.9812W

Point13	48º01.1124S	045º57.4306W

Point14	49º31.0531S	047º30.5417W

Point15	51º00.9704S	049º06.5435W

Point16	52º30.8645S	050º45.6910W

Point17	54º00.7358S	052º28.2698W

Point18	55º30.5845S	054º14.6010W

Point19	57º00.4111S	056º05.0479W

Point20	58º30.2161S	058º00.0234W

Point21	60º00.0000S	060º00.0000W

Set 11 Long Diagonal (60ºS, 30ºW to 30ºS, 60ºW)

Point1		60º00.0000S	030º00.0000W

Point2		58º30.2161S	031º59.9767W

Point3		57º00.4111S	033º54.9521W

Point4		55º30.5845S	035º45.3990W

Point5		54º00.7358S	037º31.7302W

Point6		52º30.8645S	039º14.3090W

Point7		51º00.9704S	040º53.4565W

Point8		49º31.0531S	042º29.4583W

Point9		48º01.1124S	044º02.5694W

Point10	46º31.1481S	045º33.0188W

Point11	45º01.1601S	047º01.0129W

Point12	43º31.1484S	048º26.7385W

Point13	42º01.1129S	049º50.3653W

Point14	40º31.0539S	051º12.0481W

Point15	39º00.9713S	052º31.9287W

Point16	37º30.8656S	053º50.1372W

Point17	36º00.7368S	055º06.7935W

Point18	34º30.5854S	056º22.0087W

Point19	33º00.4119S	057º35.8854W

Point20	31º30.2165S	058º48.5194W

Point21	30º00.0000S	060º00.0000W

Set 12 Long Horizontal (45ºS, 60ºW to 45ºS, 30ºW)

The rhumb line runs along the 45ºS parallel.

Set 13 Long Vertical (30ºS, 45ºW to 60ºS, 45ºW)

The rhumb line runs along the 45ºW meridian.

## Symbols

### Symbol Size

| Test Reference | SymbolSize | IHO Reference | S-98 10.2.3 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of symbols in size shown in the IHO Presentation Library. |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp |  |  |  |
| Action |  |  |  |
| Perform zoom-in and zoom-out operations in each Display Category. |  |  |  |
| Results |  |  |  |
| Confirm that the symbols do not decrease in size below that shown in the IHO Presentation Library. |  |  |  |

### Display of ECDIS chart 1 symbols of correct size

| Test Reference | Chart1Symbols | IHO Reference | S-98 19.6.1 S-98 10.2.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of the check symbol of the correct size (in mm). |  |  |  |
| Setup |  |  |  |
| Use the ECDIS to access Chart 1 OR Load the exchange set Chart1 |  |  |  |
| Action |  |  |  |
| Observe the CHKSYM01 symbol within the Information about the chart display (A,B) section. |  |  |  |
| Results |  |  |  |
| Confirm that the height of the CHKSYM01 symbol is not less than 5.0mm and not greater than 5.5mm. |  |  |  |

### Size in pixels of the check symbol CHKSYM01

| Test Reference | CheckSym | IHO Reference | S-98 10.2.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of the check symbol of the correct size (in pixels). |  |  |  |
| Setup |  |  |  |
| As for test Chart1Symbols |  |  |  |
| Action |  |  |  |
| Observe the CHKSYM01 symbol within the Information about the chart display (A,B) section. |  |  |  |
| Results |  |  |  |
| Confirm that the number of pixels (lines) which comprise the vertical extent of the symbol CHKSYM01 is not less than 16. This test may be conducted by calculation based on the properties of the EUT. |  |  |  |

### Display of text at the correct size

| Test Reference | TextSize | IHO Reference | S-52 [3.1.5] |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of text within the chart display and pick report. |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp |  |  |  |
| Action |  |  |  |
| Observe the chart display. Pick a feature and observe the text within the pick report. Create a Mariner’s note with text and observe its display. |  |  |  |
| Results |  |  |  |
| Based on viewing distance specified in manufacturer manuals, confirm that for all text observed the height of upper-case characters is not less than 3.5 mm per 1 metre viewing distance |  |  |  |

### ECDIS Chart 1

| Test Reference | Chart 1 | IHO Reference | S-98 Chart 1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Access to chart 1 datasets |  |  |  |
| Setup |  |  |  |
| Load the exchange set Chart 1 |  |  |  |
| Action |  |  |  |
| Monitor the display at a viewing scale of 1:20,000 |  |  |  |
| Results |  |  |  |
| TODO: Align these tests with full S-98 text Confirm correct display of Chart 1 datasets |  |  |  |

### Display redraw

| Test Reference | Redraw | IHO Reference | S-98 19.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Display of text within the chart display and pick report. |  |  |  |
| Setup |  |  |  |
| Load the exchange set PowerUp - Select North up true motion - Select Display Category Other - Select All Independent Mariner selectors - Simulate the own ship’s movement from Micklefirth through the Mickelfirth channel and to the Mickleden TSS roundabout. |  |  |  |
| Action |  |  |  |
| Monitor the display at a viewing scale of 1:20,000 |  |  |  |
| Results |  |  |  |
| Confirm that the display redraws in less than 5 seconds for the duration of the own ship movement. Select the display of the area north of the Lowesmore Oilfield and confirm that the display redraws in 5 seconds or informs the user and retains the previous display until ready. |  |  |  |

## Units and Legend

| Test Reference |  | UnitsLegend |  | IHO Reference | S-98 9.1.5 |
| --- | --- | --- | --- | --- | --- |
| Test description |  |  |  |  |  |
| Display units and chart legend. |  |  |  |  |  |
| Setup |  |  |  |  |  |
| Load the exchange set PowerUp |  |  |  |  |  |
| Action |  |  |  |  |  |
| Select a position for display applicable chart legend |  |  |  |  |  |
| Results |  |  |  |  |  |
| As a minimum the information listed below must be presented clearly (the complete list needs not always to be shown). Examples from the dataset loaded are listed in bold text where appropriate. |  |  |  |  |  |
|  | ECDIS Legend |  | Values |  |  |
|  | Units for depth |  | m |  |  |
|  | Units for height |  | m |  |  |
|  | Note: Units for depth and height: Although the ENC Product Specification, S-101 does not allow any other than metric depths and heights, these two elements shall be stated for clarity for the Mariner. |  |  |  |  |
|  | Scale of display |  | Selected by Mariner. (The default display scale is defined by the optimum display scale) Overscale indication where appropriate. |  |  |
|  | Data quality indicator |  | a. category of zone of confidence attribute of the Quality of Bathymetric Data feature for bathymetric data. b. Quality of Non Bathymetric Data attribute (if available) for non-bathymetric data. |  |  |
|  | Note: Due to the way quality is encoded in the ENC, both values (a. and b.) shall be used. |  |  |  |  |
|  | Sounding/vertical datum |  | Sounding datum – Lowest astronomical tide Vertical datum – Mean high water springs (VERDAT attributes of individual features shall not be used for the legend). |  |  |
|  | Horizontal datum |  | HDAT subfield of the DPSM field. WGS 84 |  |  |
|  | Value of safety depth |  | Selected by Mariner (default is 30 m). |  |  |
|  | Value of safety contour |  | Selected by Mariner (default is 30 m). |  |  |
|  | Note: If the Mariner has selected a contour that is not available in the ENC and the EUT displays a default contour, both the contour selected and the contour displayed shall be quoted. |  |  |  |  |
|  | Magnetic variation |  | Value of Magnetic variation, RYRMGV and VALACM of the MAGVAR feature shall be displayed as: VALMAG RYRMGV (VALACM) For example, 4°15W 1990 (8’E) |  |  |
|  | Edition number and date of issue of the datasets currently in use |  | Edition number and date of issue of the datasets currently in use Edition Number – Issue date - |  |  |
|  | Date and number of latest update affecting chart cells currently in use |  | Issue date and update number of the last update cell update file (ER data set) applied. Issue Date – 20010409 Update Number - 0 |  |  |
|  | Projection |  | The ENC chart projection in use |  |  |
|  | WLA Method |  | The method of water level adjustment (if used) |  |  |
| In addition, the following units shall be indicated: - position; - distance; - speed. |  |  |  |  |  |

### Legend – Multiple Product Specifications.

| Test Reference | LegendItems | IHO Reference | S-98 9.1.5 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Multiple product specifications (particularly gridded data) may have an effect on the Legend and the interpretation of the requirement in S-98: This test adds those elements. - Test behaviour of the legend with multiple product specifications loaded. |  |  |  |

## Other ECDIS and Chart Related Functionality

### ECDIS Chart 1

| Test Reference |  | ChartOne |  |  |  | IHO Reference |  |  | S-52 19.6.1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| Display of ECDIS chart 1 |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  | On |
| Safety contour |  |  | 10 |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  | 8 |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  | 30 |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  | 5 |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  | On |  | Unknown |  |  |  |  | On |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  | Off |  | Text Groups |  |  |  |  | On |
| Simplified symbols |  |  | Off |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| Ensure the ECDIS has a feature to allow Chart 1 to be displayed |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Navigate to ECDIS chart 1. Compare the displayed image with the plots shown below [any other instructions for display? NFN?] Screen plots are as displayed by compilation scale, that is 1:60 000 or 1:14 000. Screen plot number 1 is 1:60 000 and all others are 1:14 000. - Two of the screen plots (numbers 11 and 13) use “Select Simplified Point Symbols” - One screen plot (number 6) use “Select Accuracy”. |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| Confirm that ECDIS chart 1 is displayed and fills the screen when first displayed Confirm that the displayed images are consistent with the plots provided Ensure Chart 1 is labelled Not For Navigation Confirm Revision information of Chart 1 – issue date and edition number should be 2.0.0 |  |  |  |  |  |  |  |  |  |  |

| Test Reference | ChartOne2 | IHO Reference | S-52 19.6.1 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Interrogation of ECDIS chart 1. |  |  |  |
| Setup |  |  |  |
| With ECDIS chart 1 displayed. |  |  |  |
| Action |  |  |  |
| Interrogate 3 symbols by cursor pick. |  |  |  |
| Results |  |  |  |
| [Requires input from the Chart 1 datasets according to S-98] Upon interrogation the description of the feature is presented. |  |  |  |

### Use of the Colour Differentiation Diagram

The following preparatory text has been brought from S-98 into S-164 and provides instructions for testers carrying out ECDIS type approval.

These procedures should be carried out prior to executing the tests on the colour differentiation diagram.

The dusk and night tables should be checked by means of the colour differentiation test diagram, which is provided as an S-101 file.

Prior to the test:

(1) The person carrying out the test should have passed the Isihara colour blindness test, or other test used to qualify bridge watch keepers, and should adapt to night viewing for 10 minutes before checking the night display;

(2) The controls should be set to their calibrated settings;

(3) While the display is off, adjust the ambient light reflected from white paper positioned on the display screen to the following values:

Colour profile	Light level

Day		200 cd/sq. m

Dusk		10 cd/sq. m

Night		darkness (the ECDIS display is the predominant light source)

Preferably use natural daylight for the day table;

The tester should also ensure that the colour differentiation diagram has the correct issue date and edition number (from IHO).

The Colour Test must be applied on the day and dusk colour tables.

Before the Colour Test diagram is used, the black-adjust symbol BLKADJ01 must be brought up on the screen and the contrast and brightness controls (or equivalent controls for an LCD) must be adjusted as follows:

1 First, set contrast to a maximum, brightness to a minimum. Look at the black-adjust symbol. Then either:

2A If the centre square is not visible, turn up the brightness until it just appears;

or

2B If the centre square is clearly visible (with contrast at maximum, brightness at minimum), turn the contrast down until the inner square disappears, then turn contrast back up until the inner square is just visible again.

(If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure).

The "black level" is then correctly set. If a brighter display is required use the contrast control, but preferably do not adjust the controls unless lighting conditions on the bridge change.

The test consists of being able to distinguish the background colours and to pick out the like foreground colours, that is to say that squares 3, 5, 11, 15, 18 and 20 all have a shallow water blue background, and that squares 3, 10 and 17 have a grey line.

| Test Reference | ColourDifferentiation | IHO Reference | S-98 19.6.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Under each of the conditions described in the introduction, display the colour differentiation test diagram for the colour profiles. Select each table in turn and ensure that: - Each foreground diagonal line is clearly distinguished from its background; and - The foreground lines representing yellow, orange, magenta (purple), green, blue and grey may be clearly identified. |  |  |  |

| Test Reference | ColourCalibration | IHO Reference | S-98 19.3.5 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| - Relocated test from S-98 on colour calibration. - Ensure the ECDIS has instructions for colour calibration, as per S-98 19.3.5 - Check colour calibration controls have a provision to return to the calibrated settings. - Check for a warning that uncalibrated settings may adversely affect visibility of information on the display. - For instance: First, set brilliance to calibration position. Look at the black-adjust symbol. Then either: - If the centre square is not visible, turn up the brilliance until it just appears; or - If the centre square is clearly visible, turn down the brilliance until the inner square disappears, then turn brilliance back up until the inner square is just visible again. (If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure.) The "black level" is now correctly set. If a brighter display is required use the brilliance control, but it is better not to re-adjust the controls unless lighting conditions on the bridge change. Note that the black-adjust symbol should be displayed to check that the inner square remains visible on the following occasions: - Every time that the brightness or contrast controls are adjusted. - Every time that the display is switched to the night colour table. |  |  |  |

### Data Quality Indicators and Portrayal

| Test Reference | DataQualityPortrayal | IHO Reference | S-98 12.10.10 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| - Test for existence of Data Quality selector. - Test that Data quality is taken into consideration, can be configured, and meets the S-98 requirement, particularly when moving between different Data Quality areas. - Test that different combinations of data quality portrayal behave correctly. - Include all situations from Figure 11.1 in S-101PS. - Vertically overlapping QoBD features. These tests test horizontal quality portrayal. The use of vertical uncertainty in safety contour and water level adjustment processing is included in the WLA tests later |  |  |  |
| Setup |  |  |  |
| Action |  |  |  |
| Results |  |  |  |

### Miscellaneous encoding

| Test Reference | ENC encoding | IHO Reference | S-98 DC Annex S-101PC S-100 Part 10a Part 10b |
| --- | --- | --- | --- |
| Test description |  |  |  |
| TODO: Complete lists of tests from S-98 relating to dataset encoding. - Tests for complex portrayal of ENC – from portrayal catalogue. - Long record test for ISO8211 00000 header - Multiple spatial associations (in pick reports) - Composite curves and others as multiple records/repeated subfields - Exhaustive ISO8211 tests according to S-101’s profile of Part 10a - Portrayal of Shared Edges - GML encoding as per S-98 Data Constraints Annex |  |  |  |