S-164 – Part 5

Detection and Notification of Navigational Hazards

Table of Contents

Pages

5.1	Detection and Notification of Navigational Hazards - Basic test	1

5.1.1	User selection of prohibited areas	33

5.2	Detection and Notification of Navigational Hazards – Use of largest scale available	34

5.3	Detection and Notification of Navigational Hazards – Basic test Monitoring Mode	37

5.4	Detection and Notification of Navigational Hazards – Use of largest scale available – Monitoring Mode	39

5.5	Use of Data Quality Information.	42

Page intentionally left blank

## Detection and Notification of Navigational Hazards - Basic test

| Test Reference |  | NavigationalHazards |  |  |  | IHO Reference |  |  | S-98 12.10.6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication when the Mariner plans a route closer than a user-specified distance from any features satisfying the conditions for this test as listed in the S101 alert and indications catalogue and included in the test dataset 10100AA_NAVHZ.000. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the test cell 10100AA_NAVHZ.000, manually creating a route connecting all way points between features marked as WP1 through WP18 and checking display against the corresponding graphical plot View dataset 101AA00NAVHZ.000 from exchange set NavigationalHazards - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Set Plain Boundaries to On - Set Simplified symbols to Off - Select all Text groups - Manually create a route connecting all way points between features marked WP1 through WP18 Set user-specified distance for indication navigational hazards as 0.1 NM |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Check ENC symbols shown in the ECDIS against the corresponding graphical plot. Repeat sequentially with a Safety contour value of 0m, 2m, 4m, 5m, 6m, 8m, 9m, 10m, 11m, 16m, 21m, 31m, 42m, 50m, 51m |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below. Note: To increase the prominence of dangers in unsafe waters it is permitted to highlight features with an isolated danger mark when they are wholly located in this area Safety contour = 0 m, Alternative 1 |  |  |  |  |  |  |  |  |  |  |

Safety contour = 0 m, Alternative 2

Safety contour = 2 m, Alternative 1

Safety contour = 2 m, Alternative 2

Safety contour = 4 m, Alternative 1

Safety contour = 4 m, Alternative 2

Safety contour = 5 m, Alternative 1

Safety contour = 5 m, Alternative 2

Safety contour = 6 m, Alternative 1

Safety contour = 6 m, Alternative 2

Safety contour = 8 m, Alternative 1

Safety contour = 8 m, Alternative 2

Safety contour = 9 m, Alternative 1

Safety contour = 9 m, Alternative 2

Safety contour = 10 m, Alternative 1

Safety contour = 10 m, Alternative 2

Safety contour = 11 m, Alternative 1

Safety contour = 11 m, Alternative 2

Safety contour = 16 m, Alternative 1

Safety contour = 16 m, Alternative 2

Safety contour = 21 m, Alternative 1

Safety contour = 21 m, Alternative 2

Safety contour = 31 m, Alternative 1

Safety contour = 31 m, Alternative 2

Safety contour = 42 m, Alternative 1

Safety contour = 42 m, Alternative 2

Safety contour = 50 m, Alternative 1

Safety contour = 50 m, Alternative 2

Safety contour = 51 m, Alternative 1

Safety contour = 51 m, Alternative 2

### User selection of prohibited areas

| Test Reference | UserSelectionProhibited Areas | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| - From PC (Github Isse 136) - Need to test selectors exist for each individual type of prohibited area. |  |  |  |

## Detection and Notification of Navigational Hazards – Use of largest scale available

| Test Reference |  | NavigationalHazardsLS |  |  |  | IHO Reference |  |  | S-98 12.10.6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of navigational hazards. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the TDS 101AA00OVRVU.000 and 101AA00NAVHZ.000, manually creating a route connecting all way points between features marked as WP1 through WP8 and checking display against the corresponding graphical plot .. Load the exchange set NavigationalHazards and the exchange set NavigationalHazardsOverview - Select Display Category Other - Set the Safety contour value to 30 m - Set the Safety depth value to 30 m - Set Plain boundaries to On - Set Simplified symbols to Off - Select all Text groups to On |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Select position 39°57.000’N 104°49.000’W at optimum display scale (1:350 000) of 101AA00OVRVU. 1) View chart before route planning. 2) Manually create a route connecting all way points between features marked WP1 through WP8. Set user-specified distance for indication navigational hazards as 0.5 NM. Check ENC symbols shown in the ECDIS against the corresponding graphical plot |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below 1) Situation before route planning. Chart 101AA00OVRVU displayed as it is TBD 2) Situation after route planning. Alerts indicated from largest scale available for each location TBD |  |  |  |  |  |  |  |  |  |  |

## Detection and Notification of Navigational Hazards – Basic test Monitoring Mode

| Test Reference |  | NavigationalHazardsMon |  |  |  | IHO Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication if, continuing on its present course and speed, over a specified time or distance set by the Mariner, own ship will pass closer than a user-specified distance from any features satisfying the conditions for this test (as listed in section 10.5.9 of IHO S-52 and included in the test cell 101AA00NAVHZ.000) that is shallower than the Mariner's safety contour. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the test cell 101AA00NAVHZ.000, sailing with a simulated ship over the test area, setting the Safety contour to the appropriate values (0m, 2m, 5m, 6m, 8m, 9m, 10m, 11m, 16m, 21m, 31m, 42m, 50m, 51m) and checking display against the graphical plots of test 5.1 (Route plan) corresponding to each set of Safety contour settings As for test NavigationalHazards. |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Select all Text groups Check ENC symbols shown in the ECDIS for each Safety contour setting against the corresponding graphical plot. |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test 5.1. An example with Safety contour = 10 m. TBD |  |  |  |  |  |  |  |  |  |  |

## Detection and Notification of Navigational Hazards – Use of largest scale available – Monitoring Mode

| Test Reference |  | NavigationalHazardsMonLS |  |  |  | IHO Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of navigational hazards. This test is performed by loading the test cells 101AA00OVRVU.000 and 101AA00NAVHZ.000, manually creating a route connecting all way points between features marked as WP1 through WP8 and checking display against the corresponding graphical plot. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| Load the exchange set NavigationalHazards Load the exchange set NavigationalHazardsOverview - Select Display Category Other - Set the Safety contour value to 30 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries Set symbolized symbols to Off Select all Text groups .. |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Select position 39°57.000’N 104°49.000’W at the maximum display scale (1:350 000) of 101AA00OVRVU. Set simulated own ship for 39°49.587’N 104°54.930’W with heading set for 10.0° Select size of own ship check area as 1.0 NM width and 8.0 NM length. |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |

The ENC in the ECDIS should match the corresponding graphical plot shown below.

TBD

1) Situation before route monitoring. Chart 101AA00OVRVU displayed as it is. Presentation alternative 1

TBD

Situation before route monitoring. Chart 101AA00OVRVU displayed as it is. Presentation alternative 2

TBD

2) Situation during route monitoring. Alerts indicated from largest scale available for each location Presentation alternative 1

TBD

Situation during route monitoring. Alerts indicated from largest scale available for each location. Presentation alternative 2 Note: The parameters and shapes of the ship's check area are examples

## Use of Data Quality Information.

| Test Reference | DataQuality | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| This new test could be located in the Data Quality section or left here. - Test use of data quality mode. - Test for offset of lookahead for alerts and indications. - Also test for different CATZOC zones. |  |  |  |