S-164 – Part 6

# Detection of Areas for which Special Conditions Exist

Table of Contents

Pages

6.1	Detection of Areas for which Special Conditions Exist - Basic test	1

6.1.1	User selection of Areas with Special Conditions	4

6.2	Detection of Areas for which Special Conditions Exist - Use of largest scale available	5

6.3	Detection of Areas for which Special Conditions Exist - Monitoring Mode	7

6.4	Detection of Areas for which Special Conditions Exist - Use of largest scale available – Monitoring Mode	9

Page intentionally left blank

## Detection of Areas for which Special Conditions Exist - Basic test

| Test Reference |  | SpecialConditions |  |  |  | IHO Reference |  |  | S-98 12.10.7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication when the Mariner plans a route closer than a user-specified distance from the boundary of a prohibited area or a geographic area for which special conditions exist. The features satisfying the conditions for this test are listed the S-101 Alerts and indications catalogue and are included in the test cell 101AA00ARSPC.000. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the test cell 101AA00ARSPC.000, manually creating a route connecting all way points between features marked as WP1 through WP4 and checking display against the corresponding graphical plot Load the exchange set SpecialConditions - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Set symbolized symbols to Off - Manually create a route connecting all way points between features marked WP1 through WP4 Set user-specified distance for indication of areas with special condition as 0.1 NM |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Check ENC symbols shown in the ECDIS against the corresponding graphical plot. selecting one by one each special condition for the test |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |

The ENC in the ECDIS should match the corresponding graphical plot shown below.

Selected: Traffic separation zone

Selected: Inshore traffic zone

Selected: Restricted area

Selected: Caution area

Selected: Offshore production area

Selected: Area to be avoided

Selected: Military practice area

Selected: Seaplane landing area

Selected: Submarine transit lane

Selected: Anchorage area

Selected: Marine farm/aquaculture

Selected: PSSA (Particularly Sensitive Sea Area) tbd

### User selection of Areas with Special Conditions

| Test Reference | UserSelectionProhibited Areas | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| - From PC (Github Isse 136) - Need to test selectors exist for each individual type areas with special conditions.. [check that an S-98 requirement exists? Or is this all in the S-101PC?] |  |  |  |

## Detection of Areas for which Special Conditions Exist - Use of largest scale available

| Test Reference |  | SpecialConditionsLS |  |  |  | IHO Reference |  |  | S-98 12.10.7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of areas with special condition. This test is performed by loading the test cells 101AA00OVRVU.000 and 10100AA_ARSPC.000, manually creating a route connecting way points between features marked as WP20 and WP22 and checking display against the corresponding graphical plot |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| As for test SpecialConditions and in addition load the exchange set NavigationalHazardsOverview - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Set Plain Boundaries to Off - Set Simplified symbols to On Select all Text groups |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Select position 39°45′•000N 104°49′•000W at compilation scale (1:350 000) of 101AA00OVRVU. 1) View chart before route planning. 2) Manually create a route connecting two way points between features marked WP20 and WP22. Set user-specified distance for indication of areas with special conditions as 0.5 NM. Check ENC symbols shown in the ECDIS against the corresponding graphical plot |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below 1) Situation before route planning. Chart 101AA00OVRVU displayed as it is 2) Situation after route planning. Alerts indicated from largest scale available for each location. An example with Seaplane landing area and Marine farm/culture area as selected. TBD |  |  |  |  |  |  |  |  |  |  |

## Detection of Areas for which Special Conditions Exist - Monitoring Mode

| Test Reference |  | SpecialConditionsMon |  |  |  | IHO Reference |  |  | S-98 12.10.7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate alarm or indication, as selected by the Mariner, if, within a specified time set by the Mariner, own ship will cross the boundary of a prohibited area or area for which special conditions exist. The features satisfying the conditions for this test are listed in the Alerts and Indications section of the portrayal catalogue and are included in the test cell 101AA00ARSPC.000. This test is performed by loading the test cell 101AA00ARSPC.000, sailing with a simulated ship over the test area, selecting one by one each special condition for the test and checking display against the graphical plots of test 6.1 (Route plan) corresponding to each set of Safety contour settings |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| .. |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Check ENC symbols shown in the ECDIS for each special condition against the corresponding graphical plot |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test 6.1. An example with PSSA and Military practice area as selected. TBD |  |  |  |  |  |  |  |  |  |  |

## Detection of Areas for which Special Conditions Exist - Use of largest scale available – Monitoring Mode

| Test Reference | SpecialConditionsMonLS | IHO Reference | S-98 12.10.7 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of areas with special condition. This test is performed by loading the test cells 101AA00OVRVU.000 and 101AA00ARSPC.000, sailing with a simulated ship over the test area, selecting one by one each special condition for the test and checking display against the graphical plots of tests 6.1 and 6.2 (Route plan) corresponding to each special condition settings. |  |  |  |
| Setup |  |  |  |
| As for test SpecialConditionsLS |  |  |  |
| Action |  |  |  |
| Select position 39°45′•000N 104°49′•000W at compilation scale (1:350 000) of 101AA00OVRVU. Heading approximately 100°. Set vessel position to 39°47.877'N 104°57.590'W, heading 94.3°. Check ENC symbols shown in the ECDIS for each special condition against the corresponding graphical plot. |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test 6.1 and 6.2. |  |  |  |
| An example with Caution area, Military practice area and PSSA as selected TBD |  |  |  |