S-164 – Part 9

Dual Fuel Mode Testing

Table of Contents

Pages

9.1	Introduction	1

9.2	Data Scheming for Dual Fuel testing	1

9.3	Chart Loading and Update	1

9.3.1	Initial Loading of charts in Dual fuel mode.	1

9.3.2	Update(s) of combined exchange set.	2

9.4	Functions associated with chart display	3

9.4.1	Dual Fuel feature information	3

9.5	Detection and Notification of Navigational Hazards	3

9.5.1	Detection and Notification of Navigational Hazards – basic test	3

9.5.2	Dual Fuel Detection and Notification of Navigational Hazards – Use of largest scale available.	6

9.5.3	Detection and Notification of Navigational Hazards – monitoring mode	7

9.5.4	Detection and Notification of Navigational Hazards – use of largest scale available – monitoring mode	9

9.6	Detection of Areas for which Special Conditions Exist	12

9.6.1	Detection and Notification of Areas for which special conditions exist – basic test	12

9.7	12

9.7.1	Detection and Notification of Areas for which special conditions exist – use of largest scale available	13

9.7.2	Detection and Notification of Areas for which special conditions exist – monitoring mode	14

9.7.3	Detection and Notification of Areas for which special conditions exist – use of largest scale available – monitoring mode	15

9.8	Detection and Notification of the Safety contour	16

9.8.1	Detection and Notification of the safety contour – Basic test	16

9.8.2	Detection and Notification of the safety contour – use of largest scale available.	17

9.8.3	Detection and Notification of the safety contour – use of largest scale available – monitoring mode	18

9.8.4	Detection and Notification of the safety contour – monitoring mode	19

Page intentionally left blank

## Introduction

As referenced in the previous section of this manual, during the transition period from S-57 to S-101 ENCs ECDIS will be required to support “Dual Fuel” mode. This entails display of either S-57 or S-101, or both in areas where S-101 is not available yet. The tests in this section verify the ECDIS operation when both S-57 and S-101 are in operation together.

## Data Scheming for Dual Fuel testing

In order to simplify the arrangement of test data for Dual Fuel testing, some original S-57 datasets (from IHO S-64) have been used alongside S-101 versions to create the reference TDS. The arrangement of data coverage for these tests is therefore largely unchanged. A notable exception is the data scheming for the tests for navigational hazards, safety contour detection and areas where special conditions exist. These have been created alongside the original S-57 datasets, allowing exhaustive tests to be run across both types of chart format using single routes. All data is arranged in exchange sets to allow for straightforward test setup and execution.

## Chart Loading and Update

### Initial Loading of charts in Dual fuel mode.

| Test Reference | DualFuelSimple | IHO Reference | S-98 18.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Initial import of a dual fuel exchange set. |  |  |  |
| Setup |  |  |  |
| Load exchange set DualFuelSimple |  |  |  |
| Action |  |  |  |
| Ensure exchange set is loaded. Inspect contents of System Database. |  |  |  |
| Results |  |  |  |
| The System Database should contain the following entries. ENC; Edition (EDTN); Update number (UPDN); Issue Date (ISDT). 10100AA_X0000.000; 1; 0; 20190409. 10100AA_X01NE.000; 1; 0; 20210406. GB5X01NW.000; 1; 0; 20210406. |

### Update(s) of combined exchange set.

| Test Reference | DualFuelSimpleUpdate | IHO Reference | S-98 18.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| This test verifies the ECDIS is able to load updates to Dual Fuel datasets from a combined update exchange set. [these tests are very basic and may be expanded with more comprehensive exchange sets including ones where S-57 is deleted and S-101 installed instead. Additionally, dual updates, multiple data coverage and exchange sets with multiple datasets at different MSVS will be supplied. The aim is to test all reasonable edge cases when loading and basic portrayal of dual fuel is enabled. Additionally, the viewing of extra layers on top of dual fuel displays will be tested. This will show interleaving on S-101, not on S-57 – S-98 18.2] |  |  |  |
| Setup |  |  |  |
| As per previous test DualFuelSimple |  |  |  |
| Action |  |  |  |
| Load exchange set DualFuelSimpleUpdate |  |  |  |
| Results |  |  |  |
| System Database contents should show: ENC; Edition (EDTN); Update number (UPDN); Update Application Date (UADT); Issue Date (ISDT). GB5X01NW.000; 1; 1; 20190409; 20190409. 10100AA_X01NE.000; 1; 1; 20210406; 20210406. |

## Functions associated with chart display

### Dual Fuel feature information

| Test Reference | DualFuelFeatureInformation | IHO Reference | S-98 18.2 |
| --- | --- | --- | --- |
| Test description |  |  |  |
| Cursor picking in an area of DF should result in a unified display of information.. |  |  |  |
| Setup |  |  |  |
| As per test DualFuelUpdate |  |  |  |
| Action |  |  |  |
| - Set position to (60.9277,-32.4966) - Set display scale = 45,000 - Interrogate features in display |  |  |  |
| Results |  |  |  |
| Verify the information available to the user contains information from both S-57 and S-101 sources. The pick report information should contain the following information. - DRGARE (S-57) from GB5X01NW.000 - DredgedArea (S-101) from 10100AA_X01NW.000 |  |  |  |

## Detection and Notification of Navigational Hazards

### Detection and Notification of Navigational Hazards – basic test

| Test Reference |  | NavigationalHazardsDF |  |  |  | IHO Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS operating in Dual Fuel mode provides an appropriate indication when the Mariner plans a route closer than a user-specified distance from any features satisfying the conditions for this test as listed in S-98 and the S-101 Alerts and Indications Catalogue. These are included in the TDS AA5NAVHZ.000 and 101AA00NAVHZ.000. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the dual fuel exchange set NavigationalHazards, WP1 through WP36 and checking the display against the corresponding graphical plot. Load the exchange set NavigationalHazardsDF - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Select Simplified Point Symbols = false - Select all Text groups - Manually create a route connecting all way points between features marked WP1 through WP36 Set user-specified distance for indication navigational hazards as 0.1 NM |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Check ENC symbols shown in the ECDIS against the corresponding graphical plot. Repeat sequentially with a Safety contour value of 0m, 2m, 4m, 5m, 6m, 8m, 9m, 10m, 11m, 16m, 21m, 31m, 42m, 50m, 51m. |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below. |  |  |  |  |  |  |  |  |  |  |

### Dual Fuel Detection and Notification of Navigational Hazards – Use of largest scale available.

| Test Reference | NavigationalHazardsDFLS | IHO Reference | S-101PC |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of navigational hazards. This test is performed by loading dual fuel exchange sets, manually creating a route connecting all way points between marked features and checking display against a corresponding graphical plot. The same test is run twice with different overview exchange sets comprising the smaller scale data. |  |  |  |
| Setup |  |  |  |
| (A) Load the exchange set NavigationalHazardsDF and the exchange set NavigationalHazardsOverviewDF1 - Select Display Category Other - Set the Safety contour value to 30 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Select Simplfied point symbols = false - Select all Text groups (B) Repeat test using exchange sets NavigationalHazardsDF and NavigationalHazardsOverviewDF2 |  |  |  |
| Action |  |  |  |
| For each of (1) and (2) Select position 39°57.000’N 104°49.000’W at maximum display scale (1:350 000) of 101AA00OVRVU. 1) View chart before route planning. 2) Manually create a route connecting all way points between features marked WP1 through WP8. Set user-specified distance for indication navigational hazards as 0.5 NM. Check ENC symbols shown in the ECDIS against the corresponding graphical plot. |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below. A) Situation before route planning. Chart 101AA00OVRVU displayed as it is- B) Situation before route planning. Chart AA5OVRVU displayed as it is- |  |  |  |

### Detection and Notification of Navigational Hazards – monitoring mode

| Test Reference |  | NavigationalHazardsDFMon |  |  |  | IHO Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication if, continuing on its present course and speed, over a specified time or distance set by the Mariner, own ship will pass closer than a user-specified distance from any features satisfying the conditions for this test. These are included in the test cells AA5NAVHZ.000 and 101AA00NAVHZ.000) that is shallower than the Mariner's safety contour. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the exchange set NavigationalHazardsDF, sailing with a simulated ship over the test area, setting the Safety contour to the appropriate values (0m, 2m, 5m, 6m, 8m, 9m, 10m, 11m, 16m, 21m, 31m, 42m, 50m, 51m) and checking display against the graphical plots of test NavigationalHazardsDF (Route plan) corresponding to each set of Safety contour settings As for test NavigationalHazardsDF Select all Text groups |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Check ENC symbols shown in the ECDIS for each Safety contour setting against the corresponding graphical plot |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test NavigationalHazardsDF. |  |  |  |  |  |  |  |  |  |  |

### Detection and Notification of Navigational Hazards – use of largest scale available – monitoring mode

| Test Reference |  | NavigationalHazardsDFMonLS |  |  |  | IHO Reference |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |  |  |  |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of navigational hazards in dual fuel mode. |  |  |  |  |  |  |  |  |  |  |
| Loaded Data |  |  |  |  |  |  |  |  |  |  |
| Exchange Set Name |  |  |  |  |  |  |  |  |  |  |
| Display Mode |  |  |  |  | Independent Mariner’s Selections (default=On) |  |  |  |  |  |
| Other |  |  |  |  | Accuracy |  |  |  |  |  |
| Context Parameters |  |  |  |  | Contour label |  |  |  |  |  |
| Safety contour |  |  |  |  | Highlight date dependent |  |  |  |  |  |
| Safety depth |  |  |  |  | Highlight document |  |  |  |  |  |
| Deep Contour |  |  |  |  | Highlight info |  |  |  |  |  |
| Shallow contour |  |  |  |  | Shallow pattern |  |  |  |  |  |
| Four shades |  |  |  |  | Unknown |  |  |  |  |  |
| Radar overlay |  |  |  |  | Update review |  |  |  |  |  |
| Plain boundaries |  |  |  |  | Text Groups |  |  |  |  |  |
| Simplified symbols |  |  |  |  | Chart Text |  |  |  |  |  |
| Full light lines |  |  |  |  | Important text |  |  |  |  |  |
| Ignore scale minimum |  |  |  |  | Other Text |  |  |  |  |  |
| Shallow water dangers |  |  |  |  | Names |  |  |  |  |  |
| Palette |  |  |  |  | Light description |  |  |  |  |  |
| Day |  |  |  |  | All other chart text |  |  |  |  |  |
| Date Dependent Objects |  |  |  |  | Display |  |  |  |  |  |
| Start Date |  |  |  |  | Centre |  |  |  |  |  |
| End Date |  |  |  |  | Scale |  | 1:60000 |  |  |  |
|  |  |  |  |  | Orientation |  |  |  |  |  |
| Viewing Groups (Default = On) |  |  |  |  |  |  |  |  |  |  |
| Standard Display |  |  |  |  |  | Other |  |  |  |  |
| Drying lines |  |  |  |  |  | Spot soundings |  |  |  |  |
| Buoys. Beacons, aids to navigation |  |  |  |  |  | Submarine cables and pipelines |  |  |  |  |
| Buoys, beacons, structures |  |  |  |  |  | All isolated dangers |  |  |  |  |
| Lights |  |  |  |  |  | Magnetic variation |  |  |  |  |
| Boundaries and limits |  |  |  |  |  | Depth contours |  |  |  |  |
| Prohibited and restricted areas |  |  |  |  |  | Seabed |  |  |  |  |
| Chart scale boundaries |  |  |  |  |  | Tidal |  |  |  |  |
| Cautionary notes |  |  |  |  |  | Miscellaneous (Other) |  |  |  |  |
| Ships’ routeing systems and ferry routes |  |  |  |  |  |  |  |  |  |  |
| Archipelagic sea lanes |  |  |  |  |  |  |  |  |  |  |
| Miscellaneous (Standard) |  |  |  |  |  |  |  |  |  |  |
| Chart (Standard) |  |  |  |  |  |  |  |  |  |  |
| Alert Highlights (Standard) |  |  |  |  |  |  |  |  |  |  |
| Additional |  |  |  |  |  |  |  |  |  |  |
| Setup |  |  |  |  |  |  |  |  |  |  |
| This test is performed by loading the exchange sets NavigationalHazardsOverviewDF1 and NavigationalHazardsDF, manually creating a route connecting all way points between features marked as WP1 through WP8 and checking the display against a corresponding graphical plot A) Load the exchange set NavigationalHazardsDF Load the exchange set NavigationalHazardsOverviewDF1 - Select Display Category Other - Set the Safety contour value to 30 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries Set symbolized symbols to Off Select all Text groups (B) The test should then be repeated using the exchange sets NavigationalHazardsDF and NavigationalHazardsOverviewDF2 |  |  |  |  |  |  |  |  |  |  |
| Action |  |  |  |  |  |  |  |  |  |  |
| Select position 39°57.000’N 104°49.000’W at the maximum display scale (1:350 000) of 101AA00OVRVU (or AA5OVRVU). Set simulated own ship for 39°49.587’N 104°54.930’W with heading set for 10.0° Select size of own ship check area as 1.0 NM width and 8.0 NM length |  |  |  |  |  |  |  |  |  |  |
| Results |  |  |  |  |  |  |  |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plots shown below (A). |  |  |  |  |  |  |  |  |  |  |

## Detection of Areas for which Special Conditions Exist

### Detection and Notification of Areas for which special conditions exist – basic test

| Test Reference | SpecialConditionsDF | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication when the Mariner plans a route closer than a user-specified distance from the boundary of a prohibited area or a geographic area for which special conditions exist whilst operating in Dual Fuel mode. The features satisfying the conditions for this test are listed in the alerts and indications catalogue within the S-101 Portrayal Catalogue and are included in the test cells AA5ARSPC.000 and 101AA00ARSPC.000. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the exchange set SpecialConditionsDF, manually creating a route connecting all waypoints between features marked as WP1 through WP4 and checking the display against the corresponding graphical plot Load the exchange set SpecialConditionsDF - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Set symbolized symbols to Off - Manually create a route connecting all way points between features marked WP1 through WP4 Set user-specified distance for indication of areas with special condition as 0.1 NM |  |  |  |
| Action |  |  |  |
| Check ENC symbols shown in the ECDIS against the corresponding graphical plot. selecting one by one each special condition for the test |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below. |  |  |  |
| Selected: Traffic separation zone |  |  |  |
| TBD |  |  |  |

### Detection and Notification of Areas for which special conditions exist – use of largest scale available

| Test Reference | SpecialConditionsDFLS | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of areas with special conditions whilst operating in Dual Fuel mode. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading test exchange sets, manually creating a route connecting way points between features marked as WP20 and WP22 and checking the display against a corresponding graphical plot. (A) As for test SpecialConditionsDF and in addition load the exchange set NavigationalHazardsOverviewDF1 - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Select Simplified symbols Select all Text groups (B) Repeat test using exchange sets SpecialConditionsDF and NavigationalHazardsOverviewDF2 |  |  |  |
| Action |  |  |  |
| Select position 39°45′•000N 104°49′•000W at compilation scale (1:350 000) of 101AA00OVRVU (or AA2OVRVU). 1) View chart before route planning. 2) Manually create a route connecting two way points between features marked WP20 and WP22. Set user-specified distance for indication of areas with special conditions as 0.5 NM. Check ENC symbols shown in the ECDIS against the corresponding graphical plot. |  |  |  |
| Results |  |  |  |
| The ENCs in the ECDIS should match the corresponding graphical plot shown below. |  |  |  |
| tbd |  |  |  |
| 1) Situation before route planning. Chart 101AA00OVRVU displayed as it is |  |  |  |

### Detection and Notification of Areas for which special conditions exist – monitoring mode

| Test Reference | SpecialConditionsMonDF | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate alarm or indication, as selected by the Mariner, if, within a specified time set by the Mariner, own ship will cross the boundary of a prohibited area or area for which special conditions exist whilst operating in Dual Fuel mode. The features satisfying the conditions for this test are listed in listed in the alerts and indications catalogue within the S-101 Portrayal Catalogue and are included in the test cells AA5ARSPC.000 and 101AA00ARSPC.000. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the exchange set SpecialConditionsDF, sailing with a simulated ship over the test area, selecting one by one each special condition for the test and checking display against the graphical plots of test SpecialConditions (Route plan) corresponding to each set of Safety contour settings.. As for test SpecialConditionsDF |  |  |  |
| Action |  |  |  |
| Check ENC symbols shown in the ECDIS for each special condition against the corresponding graphical plot |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test 6.1. An example with PSSA and Military practice area as selected. TBD |  |  |  |

### Detection and Notification of Areas for which special conditions exist – use of largest scale available – monitoring mode

| Test Reference | SpecialConditionsDFLSMon | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detection of areas with special condition whilst operating in Dual Fuel mode.. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading test exchange sets, sailing with a simulated ship over the test area, selecting one by one each special condition for the test and checking display against the graphical plots of tests SpecialConditionsDF and SpecialConditionsDFLS (Route plan) corresponding to each special condition settings. As for test SpecialConditionsDFLS |  |  |  |
| Action |  |  |  |
| - Select position 39°45′•000N 104°49′•000W at compilation scale (1:350 000) of 101AA00OVRVU. Heading approximately 100°. - Set vessel position to 39°47.877'N 104°57.590'W, heading 94.3°. - Check ENC symbols shown in the ECDIS for each special condition against the corresponding graphical plot - Repeat test as described in SpecialConditionsDFLS |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of tests SpecialConditionsDF and SpecialConditionsDFLS. |  |  |  |
| TBD |  |  |  |
| An example with Caution area, Military practice area and PSSA as selected |  |  |  |

## Detection and Notification of the Safety contour

### Detection and Notification of the safety contour – Basic test

| Test Reference | SafetyContourDF | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate indication when the Mariner plans a route across an own ship's safety contour whilst operating in Dual Fuel mode. The features satisfying the conditions for this test are listed in listed in the alerts and indications catalogue within the S-101 Portrayal Catalogue and are included in the TDS AA5SAFCO.000 and 101AA00SAFCO.000. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the test exchange set, manually creating a route connecting all way points between features marked as WP1 through WP4 and checking the display against the corresponding graphical plot. Load the exchange set SafetyContourDF - Select Display Category Other - Set the Safety contour value to 0 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries - Select Simplified Point Symbols = true - Select all Text groups - Select Contour label - Manually create a route connecting all way points between features marked WP1 through WP4 Set user-specified distance for detecting of Safety contour as 0.1 NM |  |  |  |
| Action |  |  |  |
| Check portrayal shown in the ECDIS against the corresponding graphical plot. Repeat sequentially for Safety contour value 0m, 6m, 11m, 13m, 43m. |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below.. |  |  |  |
| Safety contour = 0 m TBD |  |  |  |

### Detection and Notification of the safety contour – use of largest scale available.

| Test Reference | SafetyContourDFLS | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS uses the largest scale available for detecting that the route crosses an own ship’s safety contour whilst operating in Dual Fuel mode. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the test exchange sets, manually creating a route connecting way points between features marked as WP11, WP24, WP25 and WP26 and checking display against the corresponding graphical plot. The same test is run twice with different overview exchange sets comprising the smaller scale data (A) As for test SafetyContourDF and in addition load the exchange set NavigationalHazardsOverview1 - Select Display Category Other - Set the Safety contour value to 11 m - Set the Safety depth value to 30 m - Select Symbolized Boundaries Set symbolized symbols to Off Select Contour label (B) Repeat test using exchange sets SafetyContourDF and NavigationalHazardsOverview2 |  |  |  |
| Action |  |  |  |
| Select position 39°27′•000N 104°49′•000W at maximum display scale (1:350 000) of 101AA00OVRVU. 1) View chart before route planning. 2) Manually create a route connecting way points between features marked WP11, WP24, WP25 and WP26. Set user-specified distance for indication navigational hazards as 0.5 NM. Check ENC symbols shown in the ECDIS against the corresponding graphical plot. |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot shown below [Images To Follow] TBD |  |  |  |

### Detection and Notification of the safety contour – use of largest scale available – monitoring mode

| Test Reference | SafetyContourDFMonLS | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate alarm if the ship, within a specified time set by the Mariner, is going to cross own ship's safety contour whilst operating in monitoring mode. The features satisfying the conditions for this test are listed listed in the alerts and indications catalogue within the S-101 Portrayal Catalogue and are included in the TDS AA5SAFCO.000 andl 101AA00SAFCO.000. . |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the exchange set SafetyContourDFMon, sailing with a simulated ship over the test area, setting the Safety contour to the appropriate values (0m, 6m, 11m, 13m, 43m) and checking display against the graphical plots of test SafetyContourDF (Route plan) corresponding to each set of Safety contour settings Load exchange set SafetyContourDFMon |  |  |  |
| Action |  |  |  |
| Set vessel position to 39°40.522'N 105°05.654'W, heading 112°. Check ENC symbols shown in the ECDIS for each Safety contour setting against the corresponding graphical plot. |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of test 7.1 and 7.2. |  |  |  |
| TBD |  |  |  |

### Detection and Notification of the safety contour – monitoring mode

| Test Reference | SafetyContourDFMon | IHO Reference |  |
| --- | --- | --- | --- |
| Test description |  |  |  |
| The purpose of this test is to verify by observation that ECDIS provides an appropriate alarm if the ship, within a specified time set by the Mariner, is going to cross own ship's safety contour. The features satisfying the conditions for this test are listed in listed in the alerts and indications catalogue within the S-101 Portrayal Catalogue and are included in the test cells AA5SAFCO.000 and 101AA00SAFCO.000. |  |  |  |
| Setup |  |  |  |
| This test is performed by loading the exchange set SafetyContourDFMon, sailing with a simulated ship over the test area, setting the Safety contour to the appropriate values (0m, 6m, 11m, 13m, 43m) and checking display against the graphical plots of test SafetyContourDF (Route plan) corresponding to each set of Safety contour settings. - As for test SafetyContourDF - Select all Text groups - Select Contour label |  |  |  |
| Action |  |  |  |
| Set vessel position to 39°36.516'N 104°55.737'W, heading 70.3°. Check ENC symbols shown in the ECDIS for each Safety contour setting against the corresponding graphical plot |  |  |  |
| Results |  |  |  |
| The ENC in the ECDIS should match the corresponding graphical plot of SafetyContourDF |  |  |  |
| TBD |  |  |  |
| An example with Safety contour = 6 m. |  |  |  |

ANNEX A: Automatic Updates via Telecommunications networks (SECOM)

As described in the introduction to this manual, S-164 now contains specifications for tests of updates using online services implemented using the SECOM specification (IEC 63173-2).

If the EUT has implemented the capability of receiving S-124 and/or S-129 official updates via a SECOM based telecommunications network, then the following tests should be executed.

These tests are executed using the following IHO standard access details:

To Be Added:

- UserName(s)

- Addresses

- Authentication key information

Connection

| Test Reference | SECOMSelection | IHO Reference IEC Reference | S-98: 20.4.3 (i) 63173-2: |
| --- | --- | --- | --- |
| Test Description |  |  |  |
| Check SECOM identity. |  |  |  |
| Safety contourSafety depthShallow contourShallow patternFour shadesRadar overlayUpdate reviewPlain boundariesSimplified symbolsFull light linesShallow water dangersSetup |  |  |  |
| Action |  |  |  |
| Connect User to SECOM network (IHO hosted, or locally hosted depending on resources) - Connect the ECDIS to the IHO based SECOM service using the details provided in this section. - Select the update data service for S-124 and S-129 data. - Verify that the identity is correctly authenticated on the ECDIS. The details are listed below: Insert User Details available through the SECOM network. |  |  |  |
| Results |  |  |  |

Service Management

| Test Reference | SECOMManagement | IHO Reference IEC Reference |  | S-98: 20.4.3 (i) |
| --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |
| Check SECOM management. |  |  |  |  |
| Loaded Data |  |  |  |  |
| Exchange Set Name |  |  |  |  |
| Safety contourSafety depthShallow contourShallow patternFour shadesRadar overlayUpdate reviewPlain boundariesSimplified symbolsFull light linesShallow water dangersSetup |  |  |  |  |
| Action |  |  |  |  |
| Verify user is able to “manage” the SECOM connection / status [it may be that other mandatory functionality is able to be added here] - Verify SECOM status is available to the ECDIS user. - Reference details are listed below: - Ensure that an update is available to the following datasets within the System Database - 129AA00XXXX, 124AA00XXXX - [user details, history and subscription status – these should all be viewable to the user] |  |  |  |  |
| Results |  |  |  |  |

Update and Acknowledgment.

| Test Reference | SECOMAcknowledgment | IHO Reference IEC Reference |  | S-98: 20.4.3 (iii) |
| --- | --- | --- | --- | --- |
| Test Description |  |  |  |  |
| Check SECOM acknowledgment. |  |  |  |  |
| Loaded Data |  |  |  |  |
| Exchange Set Name |  |  |  |  |
| Safety contourSafety depthShallow contourShallow patternFour shadesRadar overlayUpdate reviewPlain boundariesSimplified symbolsFull light linesShallow water dangersSetup |  |  |  |  |
| Action |  |  |  |  |
| Perform and acknowledge an update to the system database through the SECOM network. - Perform an update through the SECOM network to the S-129 and S-124 datasets as listed in the resources available. - Ensure an acknowledgment of receipt and verification of data integrity is observed on the ECDIS. - Ensure the data is correctly ingested into the System Database. - Interrogate the ECDIS System Database and ensure the following revision information is correct. |  |  |  |  |
| Results |  |  |  |  |
| Dataset; Edition; Update number; Issue Date (ISDT). 129AA00XXXA; 1; 0; 20240409. 124AA00NEWA; 1; 0; 20240406. 124AA00UPDB; 1; 0; 20240406. |